# UAV Telemetry Link Monitor

Hardware Setup Guide for ESP32 LoRa Drone and Ground Station Nodes

| Field | Configuration |
| --- | --- |
| UAV platform | Parrot Anafi carrying an independent ESP32 + LoRa telemetry payload |
| System role | Drone-mounted ESP32 telemetry transmitter and USB ground station receiver |
| Radio link | 915 MHz LoRa, default SF9, 125 kHz bandwidth, coding rate 4/7 |
| Dashboard input | Ground station USB serial JSON at 115200 baud |
| Primary use | Flight-test link health monitoring: RSSI, SNR, packet loss, latency, and SF adaptation |

# 1. Drone Selection

The selected aircraft is the Parrot Anafi. The Anafi remains on its normal pilot command link and failsafe behavior, while a separate Arduino-style ESP32 LoRa node is mounted as a non-flight-critical telemetry payload.

| Decision | Selection | Reason |
| --- | --- | --- |
| UAV | Parrot Anafi | Portable field platform suitable for line-of-sight link testing without building a custom airframe. |
| Telemetry node | ESP32 + SX1276/SX1278 LoRa | Low-cost Arduino-compatible hardware with direct access to LoRa radio metrics. |
| Integration boundary | External payload | Keeps telemetry experiments separate from the Anafi flight-control system. |

Mount the payload so it does not block propellers, camera/gimbal motion, cooling vents, GPS reception, or battery access. Use independent payload power and verify aircraft balance before flight.

# 2. Required Hardware

| Item | Qty | Notes |
| --- | --- | --- |
| Parrot Anafi | 1 | Selected UAV platform carrying the independent LoRa telemetry payload. |
| ESP32 development board | 2 | One board flies on the UAV, one board stays at the ground station. |
| SX1276 or SX1278 LoRa radio module | 2 | Use modules rated for 915 MHz operation. Keep both ends on the same module family when possible. |
| 915 MHz antennas | 2 | Matched antennas are mandatory. Never transmit without an antenna attached. |
| USB cable | 1+ | Needed for flashing both ESP32 boards and for live serial telemetry from the receiver. |
| Regulated 3.3 V supply | 2 | Radio modules must use 3.3 V logic and supply. Avoid 5 V on LoRa module pins. |
| Jumper wires or custom harness | As needed | Keep SPI leads short, secure, and strain-relieved for flight use. |
| Laptop running dashboard | 1 | Runs telemetry_dashboard.py and records CSV flight logs. |

RF safety note Attach a tuned 915 MHz antenna before powering either LoRa transmitter. Running the radio without an antenna can damage the RF output stage and will make RSSI/SNR testing meaningless.

# 3. Default Pin Map

Use the same SPI and control wiring on both the drone node and the ground station unless you also change the firmware pin constants.

| LoRa Signal | ESP32 Pin | Purpose |
| --- | --- | --- |
| NSS / CS | GPIO 5 | SPI chip select for the SX127x module. |
| DIO0 | GPIO 26 | Radio interrupt line used by RadioLib receive/transmit handling. |
| RESET | GPIO 14 | Radio reset line controlled by firmware. |
| DIO1 | GPIO 33 | Secondary radio interrupt line. |
| SCK | GPIO 18 | SPI clock on the ESP32 VSPI bus. |
| MISO | GPIO 19 | SPI data from LoRa module to ESP32. |
| MOSI | GPIO 23 | SPI data from ESP32 to LoRa module. |
| 3V3 | 3.3 V | Regulated radio power. Do not use 5 V. |
| GND | GND | Shared ground between ESP32 and LoRa module. |

* If your board uses a different SPI bus or LoRa breakout pinout, update PIN_LORA_NSS, PIN_LORA_DIO0, PIN_LORA_RST, and PIN_LORA_DIO1 in both sketches. Keep SPI pins aligned with the board's SPI defaults or initialize SPI explicitly. The supplied sketches now reserve GPIO 18 for SCK and GPIO 5 for NSS/CS.

# 4. Wiring Procedure

Disconnect USB and flight power before wiring the radio modules.

Wire 3.3 V and GND first, then connect SPI SCK, MISO, MOSI, and NSS/CS.

Connect RESET, DIO0, and DIO1 to the ESP32 pins listed in the default pin map.

Attach the 915 MHz antennas to both LoRa modules before applying power.

Secure the drone-side module so the antenna connector, jumper leads, and USB/power connector cannot vibrate loose.

Power each board over USB for initial bench testing. Move to flight power only after serial logs and LoRa packets are verified.

# 5. Firmware Setup

Install the Arduino ESP32 board package and the RadioLib library, then flash each node with its matching sketch.

| Node | Sketch | Expected serial boot message |
| --- | --- | --- |
| Drone node | firmware/drone_node/drone_node.ino | {"type":"boot","role":"drone_node","freq_mhz":915,"sf":9,"bw_khz":125} |
| Ground station | firmware/ground_station/ground_station.ino | {"type":"boot","role":"ground_station","freq_mhz":915,"sf":9,"bw_khz":125} |

Both sketches default to 915 MHz, SF9, 125 kHz bandwidth, coding rate 4/7, CRC enabled, and 17 dBm transmit power. Change these values only if both ends are updated together and the setting is legal for your operating region.

# 6. Ground Station Laptop Setup

Connect the ground station ESP32 to the laptop over USB.

Confirm the COM port in Windows Device Manager.

Install the dashboard serial dependency with pip install -r dashboard\requirements.txt.

Run python dashboard\telemetry_dashboard.py --serial COM7 --baud 115200 --port 8765, replacing COM7 with the real port.

Open http://127.0.0.1:8765 and verify RSSI, SNR, latency, loss, and link-quality tiles update once packets arrive.

# 7. Bench Bring-Up Checklist

Drone node prints periodic TX records with increasing sequence numbers.

Ground station prints JSON rx records with matching sequence numbers.

RSSI is plausible at bench distance, usually much stronger than -80 dBm with antennas close by.

SNR is positive and stable during a short static test.

Dashboard shows live samples and the recent packet table reaches 12 rows.

CSV flight log appears under the selected log path and grows as packets arrive.

Moving or shielding one antenna produces visible RSSI/SNR changes, confirming the dashboard is reading live RF conditions.

# 8. Flight Installation Notes

## Antenna placement

Mount the drone antenna clear of carbon fiber, high-current ESC wiring, battery leads, and video transmitters. Keep the radiating element vertical when the ground station antenna is vertical. If the UAV banks during low-altitude passes, expect RSSI/SNR dips from polarization and multipath changes.

## Power and noise

Use a clean 3.3 V rail with enough transient current for LoRa transmit bursts. If the module browns out during transmit, symptoms often look like random packet loss or stalled sequence numbers. Add local decoupling near the radio module for flight wiring.

## Mechanical reliability

Replace loose breadboard jumpers with soldered leads, locking connectors, or a small carrier board before flight. Strain-relieve the antenna pigtail and avoid sharp bends at u.FL or SMA connectors.

# 9. Link Adaptation Behavior

The ground station recommends and transmits conservative spreading-factor changes based on rolling RSSI, SNR, latency, and packet-loss metrics. The drone accepts CFG messages and reconfigures the radio. During flight tests, record every SF change so post-flight analysis can separate RF propagation effects from configuration changes.

| Condition | Recommended response |
| --- | --- |
| Loss above 8%, SNR below 4 dB, or RSSI below -112 dBm | Increase SF by one step if not already at SF12. |
| Loss below 2%, SNR above 9 dB, RSSI above -98 dBm, and low latency | Decrease SF by one step if not already at SF7. |
| Short burst loss during low-altitude pass | Hold state until the rolling window confirms the fade is persistent. |

# 10. Troubleshooting

| Symptom | Likely cause | Check |
| --- | --- | --- |
| No ground-station packets | Mismatched frequency/SF/BW, bad SPI wiring, or no antenna | Confirm both serial boot messages show the same frequency, SF, and bandwidth. |
| RSSI stuck near noise floor | Wrong antenna band, damaged cable, or module not transmitting | Bench test at short range with known-good antennas. |
| Random resets during transmit | 3.3 V rail sag or USB power instability | Measure supply voltage near the LoRa module during transmit. |
| Dashboard connected but flatlines | Wrong COM port or non-JSON serial output | Open the serial monitor and confirm newline-delimited rx JSON records. |
| Latency spikes during fades | Multipath or burst packet loss | Compare latency trend with RSSI/SNR dips and inspect packet loss bursts in the CSV log. |

# 11. Pre-Flight Go / No-Go

Both antennas attached and mechanically secure.

Drone and ground station report the same LoRa settings.

Dashboard receives live packets for at least two minutes before launch.

CSV logging path confirmed and disk space available.

Failsafe behavior does not depend on this telemetry monitor alone.

Operating frequency and transmit power are legal for the test location.